Belt Usage Statement

SB Packing does not recommend the use of any belt other than the belts provided by us.  These belts can be identified through a brand put on the belt.  “RMS” is branded on the front of every belt sold by us.  Please contact us with any questions regarding this or for authorized belt providers.

Through extensive testing we have found that the belt we offer gives the SB Autopuller the best performance.  There are a few other companies offering what they consider “comparable” belts for the Puller.  Usage of these belts has caused some frame damage as well as production issues.  We have found the following issues with “jobber” belts.  

Slippage

This is due to a variety of differences in the belts.  One common complaint is with smooth back belts.  The belt we provide has a rough back giving it more grip to the rollers.  With the smooth back belt the roller will slip inside of the belt causing what appears to be stalling.  Another fault we have found is with the cut of the belting.  Many belts have been found to have either crooked cuts or the belts are too long or too short.  This causes misalignment of the rollers resulting in missed pulls and frame damage.  A difference of as little as ½” in the belt cut will cause misalignment of the roller and machine malfunction.  We have seen differences as much as 7” difference between belt lengths in belts provided by other belt makers.

Stiffness

Belts made with a solid track on the back of the belt do not flex as much as ours.  This causes problems with installation and adjustment.

Thickness

Changing the thickness of a belt also affects dimensions on the machine.

