Bottom Airbag Retrofit Instructions

1. Ensure you are following all plant safety precautions and procedures. 

2. Cut and Move lower maintenance pin mounts to new location shown on top view bottom airbag style frame on retrofit instruction page.

3. Disconnect and remove top airbags that are hooked to the bottom rollers via bar.

4. Remove linkage and support bar hooking the top airbags to bottom roller.

5. Remove bottom roller linkage bar wear strips.

6. Raise machine up and put on maintenance pins.

7. Cut out gussets shown on retrofit instruction page on lower frame.

8. Install bolt on bar shown at bottom of page on retro fit instruction drawing.  Bar mounts using bolts that mount two front roller pillow blocks.  See picture for detail.  

9. Install weld on bar shown on side view of lower airbag mounting.  Note dimensions.  Center of 4” bar is 4” from front of frame.

10. Install gusset on weld on bar as shown on detail of side view.

11. Install 2 new airbags and hook new hoses up to machine.

12.   Perform operational check.

Please feel free to contact SB Packing House Equipment with any questions.

Regards,

Doug Siders

402-659-2451 cell

970-867-6664  office 

[image: image1.jpg]



[image: image2.jpg]BAR MOUNTS TO
BOTTOM OF BOLTS OF
mPILLOW BLOCKS

REMOVE ANY
IASSOCIATED

GUSSETS TO BE CUT
OUT OF BOTH SIDES
ON BOTTOM FRAME





[image: image3.jpg]Retra Fit Instructions

Top Airbag Style Frame
Top View

Itema In Red To B Remaved

Bottom Arbag Style Frame
Top View

=

H
1

s
P

New Style Lower Frame Retro Fit Dimensions
New Lower Safety Bar Mounts To Be Welded.

(N i)

New Upper Airbag MTG
Bar To Be Bolted
Bottom Front Bsaringa






[image: image4.jpg]Slde View Lower Airbag Mounting

ew Lower Airbag MTG
Bar Shown Above In Red
Bar To Be Welded





